Thane Creek Mapping & Area: Map the Thane Creek mangrove/wetland zone, covering approximately 65–70 km² of creek and intertidal area, with drone-based mapping of mangrove extent, canopy condition, shoreline, mudflats, tidal channels, and vegetation zones.

| Question | Field Mission Definition |
| --- | --- |
| What are we observing? | Mangrove extent, mangrove classification, canopy density, canopy height, vegetation structure, fragmentation, regeneration, shoreline, tidal channels, mudflats, water surface, terrain/elevation, and selected environmental parameters. |
| Where? | Thane Creek mangrove zones covering Airoli–Vashi, Mulund–Bhandup, Kanjurmarg, Vikhroli, Mandale and Eastern Thane Creek/Navi Mumbai, with separate marking of water, mudflat, saltpan and non-mangrove areas. |
| With what? | IMX500 12.3 MP RGB AI Camera for visual mapping/classification; RP LiDAR for canopy height, terrain and 3D structure; Oak-D Pro for local RGB-D/3D verification; GNSS/RTK for accurate georeferencing; ground sensors for environmental parameters where required. |
| How? | Conduct planned drone flights with overlapping RGB imagery and LiDAR coverage; use Oak-D Pro for selected verification areas; collect RTK/GNSS reference points; perform ground observations for species and environmental validation; QA and back up all data after each flight. |
| At what frequency? | Baseline: complete survey once. Monitoring: repeat the same flight paths and reference locations quarterly initially; increase to monthly only if change/regeneration monitoring requires it. Environmental measurements should follow defined sampling intervals at fixed reference points. |
| How is it identified? | Every flight and observation receives a unique Survey ID + Zone ID + Flight ID + Sensor ID + Observation ID + GPS coordinates + timestamp. Example: TC-Z03-F02-IMX500-OBS001. |
| Where does it go? | Sensors → onboard storage → field SSD backup → QA → processing → GIS database + LiDAR/3D dataset + environmental database → integrated Thane Creek Mangrove Dataset → final maps/reports/change-detection dashboard. |

| Zone | Area / Location | Mangrove classification | What to mark |
| --- | --- | --- | --- |
| Z1 | Airoli–Vashi | Dense/protected mangrove + mudflat | Mangrove boundary, canopy density, height, mudflat, tidal channels |
| Z2 | Mulund–Bhandup | Dense mangrove patches | Mangrove extent, degradation/gaps, canopy condition |
| Z3 | Kanjurmarg | Large mangrove zone | Dense/sparse mangrove, open patches, drainage/tidal channels |
| Z4 | Vikhroli | Mangrove + intertidal wetland | Mangrove boundary, vegetation density, mudflat/water interface |
| Z5 | Mandale | Smaller mangrove patch | Patch boundary, canopy condition, surrounding wetland |
| Z6 | Eastern Thane Creek / Navi Mumbai side | Mangrove + intertidal/wetland patches | Mangrove extent, mudflats, tidal channels and fragmented patches |
| Z7 | Non-mangrove creek/water | Water body | Water boundary, tidal channels, depth/flow indicators |
| Z8 | Mudflat / saltpan | Non-mangrove intertidal | Bare mudflat, saltpan, colonisation by new mangroves |

200 m height recommended flights

12.3MP Sony IMX500 sensor for low res

Upgraded lens recommended for procurement

Check on geosurfacing

| Zone | Mangrove Class | IMX500 RGB Camera | RP LiDAR – Plan A | Oak-D Pro – Plan B | Drone Capability | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Z1 Airoli–Vashi | Dense mangrove | 🟢 Good | 🟢 Good | 🟡 Local 3D | Good | Map canopy extent, gaps and visible condition |
| Z2 Mulund–Bhandup | Dense/fragmented | 🟢 Good | 🟢 Good | 🟡 Local 3D | Good | RGB useful for boundary and fragmentation mapping |
| Z3 Kanjurmarg | Dense/moderate | 🟢 Good | 🟢 Good | 🟡 Local 3D | Good | Suitable for baseline mapping and repeat surveys |
| Z4 Vikhroli | Mangrove/wetland | 🟢 Good | 🟢 Good | 🟡 Local 3D | Good | Vegetation and wetland boundaries visible |
| Z5 Mandale | Fragmented/small patches | 🟢 Good | 🟡 Optional | 🟡 Good locally | Good | RGB sufficient for patch mapping |
| Z6 Eastern Creek/Navi Mumbai | Mangrove/wetland | 🟢 Good | 🟢 Good | 🟡 Local 3D | Good | Suitable for large-area aerial mapping |
| Z7 Open water/tidal channels | Water | 🟢 Good for surface | 🟢 Good for geometry | 🟡 | Partial | Surface/channel mapping only; no underwater measurement |
| Z8 Mudflat/saltpan | Non-mangrove/intertidal | 🟢 Excellent | 🟢 Good | 🟡 | Excellent | Easy RGB classification and boundary mapping |

| Observation | Required Output | Primary Equipment | Status | Capability | Remarks |
| --- | --- | --- | --- | --- | --- |
| Mangrove Extent | Mangrove boundary/area | IMX500 RGB | Available | 🟢 High | Primary mapping sensor |
| Mangrove Classification | Dense / Moderate / Sparse / Regenerating / Non-mangrove | IMX500 + AI | Available | 🟢 High | Ground validation required |
| Canopy Density | Coverage/gaps | IMX500 RGB | Available | 🟢 High | Repeatable using same flight plan |
| Canopy Height | Tree/canopy height | RP LiDAR | Available | 🟢 High | Preferred for quantitative height |
| 3D Vegetation Structure | Canopy + terrain structure | RP LiDAR | Available | 🟢 High | Primary 3D dataset |
| Local 3D Verification | RGB-D/depth structure | Oak-D Pro | Available | 🟡 Medium | Use for selected areas/ground validation |
| Visual Mangrove Health | Visible stress/dieback | IMX500 RGB | Available | 🟢 Medium–High | Visual assessment only |
| NDVI / Spectral Health | Vegetation indices | Multispectral | Not available | 🔴 None | Do not claim NDVI from IMX500 |
| Fragmentation | Patch gaps/isolation | IMX500 RGB | Available | 🟢 High | AI/CV classification |
| Shoreline | Mangrove-water boundary | IMX500 + LiDAR | Available | 🟢 High | Combine imagery + elevation |
| Tidal Channels | Channel geometry | IMX500 + RP LiDAR | Available | 🟢 High | Surface geometry only |
| Mudflat | Mudflat extent | IMX500 RGB | Available | 🟢 High | Easy land-cover classification |
| Saltpan/Open Land | Non-mangrove classification | IMX500 RGB | Available | 🟢 High | Separate land-cover class |
| Water Surface | Creek/water boundary | IMX500 RGB | Available | 🟢 High | Surface mapping only |
| Surface Elevation | DEM/terrain | RP LiDAR | Available | 🟢 High | Requires accurate georeferencing |
| Regeneration | New mangrove patches | IMX500 RGB + Oak-D | Available | 🟡 Medium–High | Ground verification required |
| Species Identification | Species classification | IMX500 + AI | Available | 🟡 Partial | Ground truth required |
| Water Quality | pH/salinity/DO/turbidity | Ground sensors | Gap | 🔴 Drone cannot measure | Procure only if mission requires |
| Soil/Sediment | Soil properties | Ground sampling | Gap | 🔴 Drone cannot measure | Ground sampling required |
| Change Detection | Area/condition change | IMX500 + RP LiDAR | Available | 🟢 High | Requires repeat surveys |

Minimum field rule

Do not leave the survey location until:
Imagery ✓ + LiDAR ✓ + Oak-D ✓ + GPS ✓ + Mission Log ✓ + Backup ✓

GIS + 3D + ENVIRONMENTAL DATA

1. GIS DATA

IMX500 RGB imagery → Georeferencing → GIS classification

Generate:

- Mangrove boundary

- Dense mangrove

- Moderate mangrove

- Sparse/degraded mangrove

- Regenerating mangrove

- Non-mangrove

- Mudflat

- Saltpan/open land

- Water body

- Shoreline

- Tidal channels

- Survey/ground-reference points

2. 3D DATA

RP LiDAR → Point Cloud → Terrain/Canopy Processing

Generate:

- Raw point cloud

- Classified ground/non-ground points

- Digital Terrain Model (DTM)

- Digital Surface Model (DSM)

- Canopy Height Model (CHM)

- Mangrove height distribution

- 3D vegetation structure

Oak-D Pro → RGB-D → Local 3D verification

Use Oak-D primarily for selected ground/low-altitude validation areas, rather than treating it as the main large-area LiDAR replacement.

Complete data pipeline

IMX500 RGB
↓
Orthomosaic + Mangrove Classification
↓
GIS Mangrove Map

RP LiDAR
↓
Point Cloud + DTM/DSM/CHM
↓
3D Mangrove Structure

Oak-D Pro
↓
RGB-D Local Validation
↓
Ground/Local 3D Dataset

Final Integrated Dataset

GIS = Where it is
3D = What its physical structure is
Environmental data = What its surrounding conditions are
Time/Survey ID = When it was observed

This gives you one integrated Thane Creek Mangrove Field Dataset, rather than three disconnected datasets.